# OverheatLens: a local-first, validation-first platform for domestic overheating assessment with versioned overheating standards

4 September 2026

# Summary

OverheatLens is an open, local-first research platform for domestic overheating assessment. It chains the full evidence workflow — weather-file quality control, building-model readiness checks, EnergyPlus simulation, versioned overheating standards, thermal-comfort analytics, mitigation experiments and a reproducible evidence archive — behind a single local service with a laboratory-style web interface. Its distinguishing design rule is traceability: every number shown in the interface carries provenance (weather-file hash, model hash, rule-pack version, EnergyPlus version and a run identifier), and every archived run can be replayed or exported as a reproducibility bundle.

# Statement of need

Domestic overheating is a growing risk in temperate climates, and the United Kingdom now has two operational assessment methodologies — CIBSE TM59 (Chartered Institution of Building Services Engineers (CIBSE) 2017, 2026) and the dynamic route of Approved Document O (Ministry of Housing, Communities and Local Government 2021) — plus the wider TM52 framework (Chartered Institution of Building Services Engineers (CIBSE) 2013) they derive from. In practice these assessments are produced with commercial graphical tools whose criteria implementations are not inspectable, whose standards versions are implicit, and whose outputs are difficult to audit or reproduce. Researchers comparing archetypes, weather years or mitigation strategies therefore struggle to answer a simple question: which definition of the standard produced this number?

OverheatLens addresses that gap. All criteria are transcribed into versioned rule packs that record the SHA-256 hash of the official PDF they were verified against, and machine-verification evidence documents that each implemented limit appears in that document. The simulation engine is the official EnergyPlus binary (Crawley et al. 2001), driven in isolated run directories with an error interpreter and a harvest step that preserves full zone keys (level:room) and hourly resolution. Comfort indices are never reimplemented: PMV/PPD, adaptive comfort and UTCI come from pythermalcomfort (Tartarini and Schiavon 2020), which wraps ISO 7730 (ISO 7730 — Ergonomics of the Thermal Environment 2025) and EN 16798-1 (EN 16798-1 2019), with the UTCI operational procedure of Bröde et al. (Bröde et al. 2012).

The platform is built for researchers who need to trust, reproduce and re-use their overheating evidence rather than re-type it from a GUI report.

# Key features

- Weather intelligence. Parsing of standard and CIBSE-variant EPW files, a calibrated QC battery (range, sentinel, dew-point physics, discontinuity, stuck-sensor and duplicate checks) with PASS / PASS_WITH_WARNINGS / FAIL classification, headline metrics, and standards-compatibility review (e.g. TM59:2017 and TM59:2026 weather requirements differ, and the app states which files satisfy which).

- Model readiness. Object-level IDF inspection before any simulation: zone inventory, occupant and glazing census, and standards-specific checks with an explicit reason for every finding.

- Versioned standards. TM59:2017 (adaptive criterion a with the published occupancy bases, 32-hour sleep-window criterion b), TM59:2026 (criteria a–d with night-based criterion b and a communal 28 °C criterion), Approved Document O dynamic route, and TM52 criteria 1–3 — each evaluated with exact published boundaries and explicit NOT_EVALUATED honesty states rather than invented numbers.

- Thermal comfort. PMV/PPD, EN 16798-1 adaptive acceptability and UTCI, computed by the wrapped library on the simulated operative temperature, with assumptions recorded in every result.

- Mitigation experiments. Real paired EnergyPlus experiments (baseline versus stored strategy variants on a chosen weather file) reporting overheating verdicts, comfort and annual facility energy; cases that cannot be computed are reported as INCOMPLETE, never estimated.

- Evidence archive. Every run is persisted with manifest, series, criteria tables and provenance; any run can be exported as a reproducibility bundle.

# Validation

A 16-case scientific validation campaign ships with the repository and runs in about fifteen minutes on a laptop. It covers the rule-pack source chain, real weather-file QC, verdict-flip exactness at published TM59:2017, TM59:2026, TM52 and Part O boundaries, PMV/PPD against the published ISO 7730 anchors, UTCI against the Bröde et al. reference conditions, EnergyPlus determinism, facility meter internal consistency, a zone-by-zone cross-check against DesignBuilder results for the same measured dwelling, and three-way equality between the EnergyPlus output file, the on-disk archive and the numbers served to the interface. The campaign reports PASS, FAIL or INCOMPLETE and its results are rendered inside the application’s validation page.

# Quality of tests and ongoing development

The repository carries 147 core, 34 API and 12 interface tests, all runnable locally without cloud services (Run Tests launchers are provided), plus a GitHub Actions workflow for continuous integration. Governance documents (contribution guide, code of conduct, security policy, disclaimer and governance model) are included; the project explicitly labels research-only outputs and refuses to present them as compliance certificates. Development is active, with the validation campaign designed to grow alongside the standards it tracks.

# References

Bröde, Peter, Dusan Fiala, Krzysztof Blazejczyk, et al. 2012. “Deriving the Operational Procedure for the Universal Thermal Climate Index (UTCI).” International Journal of Biometeorology 56 (3): 481–94. https://doi.org/10.1007/s00484-011-0454-1.

Chartered Institution of Building Services Engineers (CIBSE). 2013. TM52: The Limits of Thermal Comfort — Avoiding Overheating in European Buildings. CIBSE.

Chartered Institution of Building Services Engineers (CIBSE). 2017. TM59: 2017 — Design Methodology for the Assessment of Overheating Risk in Homes. CIBSE.

Chartered Institution of Building Services Engineers (CIBSE). 2026. TM59: 2026 — Design Methodology for the Assessment of Overheating Risk in Homes. CIBSE.

Crawley, Drury B., Linda K. Lawrie, Frederick C. Winkelmann, et al. 2001. “EnergyPlus: Creating a New-Generation Building Energy Simulation Program.” Energy and Buildings 33 (4): 319–31. https://doi.org/10.1016/S0378-7788(00)00114-6.

EN 16798-1:2019 — Energy Performance of Buildings. Ventilation for Buildings. Part 1: Indoor Environmental Input Parameters for Design and Assessment of Energy Performance of Buildings Addressing Indoor Air Quality, Thermal Environment, Lighting and Acoustics (2019).

ISO 7730 — Ergonomics of the Thermal Environment: Analytical Determination and Interpretation of Thermal Comfort Using Calculation of the PMV and PPD Indices and Local Thermal Comfort Criteria (2025).

Ministry of Housing, Communities and Local Government. 2021. The Building Regulations 2010 — Approved Document o: Overheating. HM Government.

Tartarini, Federico, and Stefano Schiavon. 2020. “Pythermalcomfort: A Python Package for Thermal Comfort Research.” SoftwareX 12: 100578. https://doi.org/10.1016/j.softx.2020.100578.